- Control Board

- Addr

This function is used to set or adjust the fixture’s starting DMX address. Every device controlled by DMX has to have a unique starting address. The addressing feature is what allows DMX to function properly. The DMX address of a fixture is what allows it to communicate with a controller properly.

- PASS

Password is the console control motor rotation required, ±2 effective.

For example, the password value is 100, to control the motor, the password value range is 98-102.

- ModE

Mo.00  DMX control mode

Mo.01  Remote control mode

- DMX Channel(Mo.00)

| 4 Channel Mode |  |  |
| --- | --- | --- |
| Channel | Value | Function description |
| CH1 | 0-64 | Stop rotation |
|  | 65-128 | Forward rotation |
|  | 129-192 | Stop rotation |
|  | 193-255 | Reversal rotation |
| CH2 | 0-255 | No Function |
| CH3 | 0-255 | Speed(slow --> fast) |
| Ch4 | 0-255 | The values “PASS” on the display panel(±2 Valid value) |

- Remote control Board(Mo.01)

- Key1:Stop rotation

- Key2:Forward rotation

- Key3:Reversal rotation

- Key4-Key12:Rotation speed,from slow to fast

- (Key4:The slowest speed; Key12:The fastest speed)